FT991A Rig Control Version 1c — Setup & User Guide

Version 1b Notice: This is Version 1 build shared for testing purposes. Please report any bugs, unexpected behavior, or suggestions. Your feedback directly shapes the next release.

# 1.  What’s in the Folder

| File | Purpose |
| --- | --- |
| FT991A_RigControlV1c.exe | The program. Double-click to run — no installer needed. |
| FT991A_Setup.docx | This document. |
| 991a_Control_Settings.json | Created automatically on first run. Saves your COM port, baud rate, and slider positions between sessions. |
| 991a_memories.json | Created automatically on first run. Stores your 30 memory / favorite slots. |
| 991a_log.csv | Created automatically on first run. Rolling 30-day log of all events, errors, and CAT traffic (when enabled). |
| FT991A_Tips.docx | Optional tips and tricks document. |

Tip: Keep all files together in the same folder. The program reads and writes 991a_Control_Settings.json, 991a_memories.json, and 991a_log.csv from whichever folder the .exe lives in.

# 2. Hardware & Driver Setup

Connect the radio to your PC with a USB cable before launching the program. Windows should install the Silicon Labs CP210x driver automatically. Most people are using a USB 2.0 printer cable.

## 2.1 Verify the COM Port in Device Manager

Make sure the cable is connected and the radio is on. Open Windows Device Manager and expand Ports (COM & LPT). Look for:

Look for: Silicon Labs CP210x USB to UART Bridge "Enhanced" COM Port (COM X) — write down the COM number.

The Enhanced port is the CAT control port. There is also a Standard port — do not use it for this program.

## 2.2 Disable Power Management on the COM Port

Windows can turn off USB devices to save power, which drops the CAT connection mid-session. Prevent this:

- Right-click on the Enhanced COM Port entry.

- Select Properties, then click the Power Management tab.

- Uncheck “Allow the computer to turn off this device to save power.”

- Click OK.

## 2.3 Verify COM Port Speed Settings

Right-click the Enhanced COM Port → Properties → Port Settings tab. Confirm these values:

| Setting | Required Value |
| --- | --- |
| Bits per second | 38400 |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |

## 2.4 Verify Audio Device

In Device Manager expand Sound, Video and Game Controllers. Confirm you see:

Look for: USB Audio CODEC — this confirms the radio’s audio interface is active.

## 2.5 Check for Conflicts

Before launching, confirm in Device Manager:

- No duplicate COM port entries

- No disabled devices (shown as greyed-out)

- No yellow warning triangles on any USB or COM entry

## 2.6 Configure Radio CAT Settings

On the FT991A, press MENU then select SETUP. Navigate to the following two menu items and set them as shown. These settings must match the program or the connection will fail.

| Menu Item | Setting | Required Value |
| --- | --- | --- |
| 031 CAT RATE | CAT baud rate | 38400 bps |
| 032 CAT TOT | CAT timeout | 1000 msec |

# 3.  First Launch

- Double-click FT991A_RigControl_V1.exe. No installer is required.

- In the Live (Selected / Unselected VFO) panel at the top left, select your COM port from the dropdown (the Enhanced port you noted earlier).

- Set Baud to 38400.

- Make sure the RTS checkbox is checked.

- Click Connect. The status label will change to Connected: COMX when successful.

Note: If the program was run before with a saved COM port, it will attempt to auto-connect on startup. If the radio is on and the cable is connected, it connects automatically with no action needed.

# 4. Program Layout

The window is divided into two columns. Everything on the left is live radio status and VFO/memory control. Everything on the right is for setting targets, levels, and diagnostics.

## Left Column

| Panel | What it does |
| --- | --- |
| Live (Selected / Unselected VFO) | Shows the real-time frequency, mode, and band for both VFOs. Also contains the COM port connection controls, S-meter bar, and RX/TX button. |
| VFO Control | Select VFO A or B as the active receiver. Swap A↔B exchanges both VFOs. Copy A→B copies VFO A’s frequency and mode into VFO B. |
| Memories (Favorites) | 30 user-programmable memory slots. Save a frequency/mode/band/filter combo, recall it to the radio, or delete it. The list shows all named slots. |
| Split | Enable split TX/RX operation, set a TX offset, and monitor RF and TX frequencies. XFC (cross-VFO check) momentarily tunes the receiver to the TX frequency. |

## Right Column

| Panel | What it does |
| --- | --- |
| Frequency / Mode / Band | Type in a band, frequency, and mode then click Set Freq, Set Mode, Set Band, or Set All to send them to the active VFO. The ▲/▼ buttons and step selector let you fine-tune the frequency in small increments. |
| Levels | Real-time sliders for Volume, Squelch, RF Power, Mic Gain, and RF Gain. Changes are sent live as you drag. |
| TX Meters | Power, SWR, and ALC bars update while transmitting. |
| Quick Buttons | One-click DATA, RTTY, SSB, and CW mode buttons. Each click cycles between upper and lower variants (e.g. USB ↔ LSB, CW-U ↔ CW-L). |
| Frequency Scanner | Scans a frequency range on the active VFO, pausing when signal strength meets the Stop-at threshold. Start, Pause, and Stop controls provided. |
| TX Timeout Safety | Automatically drops MOX if the radio transmits longer than the configured timeout. Enable the checkbox and set the seconds. |
| Log | On-screen event log with diagnostic toggles (see Section 6). |

# 5. VFO Controls

The VFO Control panel has three buttons.

| Button | What it does |
| --- | --- |
| Swap A--B | Exchanges the frequency and mode of both VFOs on the radio, then refreshes the display. |
| Copy A--B | Copies VFO A’s frequency, band, and mode into VFO B, then refreshes the display. |
| Copy B--A | Copies VFO B’s frequency, band, and mode into VFO A, then refreshes the display. |

After Scanning: If you use the Frequency Scanner and stop on a signal, use Swap A--B or Copy A--B afterward to set your desired VFO to the active frequency.

# 6. Log & Diagnostics

The Log panel shows timestamped events in the on-screen text box and writes them to 991a_log.csv (kept for 30 days). Four buttons control what gets logged:

| Button | What it logs |
| --- | --- |
| Log TX Meters | Logs raw RM meter readings (power, SWR, ALC) and TX/MOX state every 1.5 seconds while transmitting. Off by default. Turn on when debugging TX issues. |
| Log CAT Traffic | Logs every CAT command sent to the radio and the reply received, e.g. FA; → FA007204000;. Use this when something is behaving unexpectedly to see exactly what is happening on the serial port. Off by default — generates a lot of entries. |
| Log Heartbeat | Logs a ‘Polling heartbeat’ line every 5 seconds to confirm the background poll loop is running. Useful if the display appears frozen. Off by default. |
| Clear Log | Clears the on-screen log text box only. Does not delete the 991a_log.csv file. |

When reporting a bug: Turn on Log CAT Traffic, reproduce the problem, then open 991a_log.csv and paste the relevant lines into your bug report. This is the fastest way to diagnose what went wrong.

# 7. Troubleshooting

## Won’t Connect / “Disconnected”

- Verify you selected the Enhanced COM port (not Standard).

- Confirm baud is 38400 and RTS is checked.

- Check Device Manager for yellow warning icons on the COM port.

- Disable power management on the COM port (see Section 2.2).

- Try unplugging and re-plugging the USB cable, then refreshing the port list.

- Make sure no other program (HRD, WSJT-X, etc.) has the port open.

## Display Freezes / Stops Updating

- Enable Log Heartbeat. If heartbeat entries stop appearing, the poll loop has crashed — disconnect and reconnect.

- Check the Log panel for a “Polling stopped” error message.

- Ensure no other CAT program is competing for the same COM port.

## Frequency Not Matching After Scan

- Click Swap A--B or Copy A--B after stopping a scan to move the found frequency to your desired VFO.

## Frequency Not Changing

- Check that the Frequency / Mode (TARGET) section has the correct frequency entered, then click Set All.

- If the radio is in Split mode, the inactive VFO may be what is being written. Disable Split first.

## Settings Not Saving Between Sessions

- Confirm 991a_Control_Settings.json is in the same folder as the .exe and is not read-only.

- Click Set Defaults before closing the program to force a save.

## TX Timeout Tripping Unexpectedly

- Check the TX Timeout Safety panel. Increase the timeout seconds or uncheck Enable TX Timeout if you are doing long digital transmissions.

# 8. Version 1  Feedback

Thank you for testing! When you find something to report, the most useful information to include is:

- What you were doing when the problem occurred (e.g. stopped a scan, clicked Swap A--B).

- What you expected to happen.

- What actually happened.

- The relevant lines from 991a_log.csv (enable Log CAT Traffic before reproducing the issue for maximum detail).

- Your radio firmware version if known (shown on the FT991A front panel menu).

- If you need help, email me at 5xjranch@gmail.com

Known Limitation: There is a brief ~0.3 second delay when switching VFOs using the Select buttons. This is intentional — the program reads the VFO, switches, and re-applies the frequency and mode to guarantee audio follows the selection. It will be optimized in a future release.